# Wash Station Executive Summary

## Description:

Anybody determined to be contaminated with radioactive material will be escorted to the Wash Station for decontamination.  The decontamination process may be as minimal as removing contaminated clothing and performing localized washing or it may be as extensive as showering multiple times.  Wash Station staff determine the appropriate method of decontamination and assist in the cleaning process.

## Location:

Facilities with showers or locker rooms are ideal for incorporating a Wash Station.  Alternatively, mobile decontamination units may be employed to provide wash capabilities.

Key considerations for establishing the Wash Station include:

- ability to accommodate males and females while maintaining privacy,

- ability to accommodate families and others with special needs,

- direct access from the Contamination Screening Station,

- unimpeded access to Registration, and

- close proximity to the Initial Sorting Station.

## Staffing:

Staffing plans must accommodate the need for gender-specific wash locations.  Staff size will vary according to the size of the community reception center (CRC), the desired throughput, and the availability of professionals and trained volunteers.  Qualified and trained radiation protection professionals, public health staff, emergency services personnel, and volunteers may be needed.

Personal Protective Equipment (PPE):

The CRC Safety Officer will determine the appropriate PPE for staff in each area.  The Safety Officer will also evaluate the need for—and issue as appropriate—personal dosimetry devices among CRC staff.

Typically, community reception centers will not be located in contaminated areas.  Nevertheless, many people reporting to the CRC could be contaminated with radioactive material on their clothes or bodies, presenting a possibility of cross contamination and a potential inhalation hazard to CRC staff.  Standard medical precautions, including gown, gloves, facemask, eye shield, and appropriate respiratory protection (as determined by the CRC Safety Officer) provide adequate protection, although additional splash protection and water resistant clothing (e.g., rubber boots) may be necessary for staff assisting in the shower area.

Staff should try to minimize physical contact with people, and gloves should be changed or checked for contamination frequently.  Contamination screenings and PPE exchange will be necessary for all staff leaving the area for breaks or at shift change.  No food or drink should be consumed in this area.  Public health officials should consult their state or local radiation control authorities for assistance in developing PPE packages.

## Radiation Detection Equipment:

Wash Station staff should keep in mind that wet decontamination may mask alpha emissions and make detection of residual contamination difficult.  Health physicists or radiation protection professionals should to supervise or assist staff in this area.

Using headphones in conjunction with handheld detection instruments can help to reduce anxiety among people being screened and can improve the operator’s ability to recognize elevated radiation levels.  The type of radiation detection equipment used must be appropriate to the type of radioactive material present, as determined by radiation control authorities.